L1. The Microstructure of Materials

Using the given information, identify the microconstituents of the following metals.

Justify you answer.

|  | Pure iron (99.9%Fe, 0.1%impurities). Annealed. Microconstituents: Ferrite (alpha-Fe) Pure iron+room temperature=> BCC structure=> ferrite (single phase solid solution) |
| --- | --- |
|  | Bronze (alloy 95%Cu + 5%Sn). As cast. Microconstituents: alpha-phase (solid solution of Sn in Cu) = eutectoid structure As cast + bronzed => dendritic alpha-phase grains |
|  | Brass (alloy 70%Cu + 30%Zn). Annealed. Microconstituents: alpha-Brass (solid solution of Zn in Cu) Single-phase FCC and annealing twins (recrystallization) |
|  | Carbon steel (99.98%Fe+0.02%C). Annealed. Microconstituents: Ferrite Very low C content => ferrite BCC solution of C in Fe |
|  | Titanium alloy (TiTaZrNbAg). As cast. Microconstituents: complex solid solution and intermetallics Multi-elements => different phases but mainly solid solution strengthening |
|  | White cast iron (4.5%C+rest Fe). As cast. Microconstituents: cementite + pearlite/ledeburite White cast has cementite (hard & brittle) and an eutectic mix of austenite and cementite |
|  | Steel (0.6%C + rest Fe). Annealed. Microconstituents: pearlite and ferrite Ferrite grains |
|  | Steel (0.8%C + rest Fe). Annealed. Microconstituents: pearlite Eutectoid steel => pearlite (fine mix of ferrite & cementite) |
|  | Stainless steel (18%Cr+8%Ni+4%Mo+rest Fe). Annealed. Microconstituents: austenite Austenitic stainless steel (FCC) stabilized by Ni & Cr |